How to Read Blueprint Numbers

(The following is quoted from:  Canadian Forces, “Part List Expeditor”, 05 Jan 70, pg 4)

General

Basic part or drawing numbers consist of six original digits; essentially two digits to designate basic model, one digit to designate major group or section and three additional serial digits.

For example, 644-189300 is decoded as:

6	-	Type

4	-	Class

4	-	Engine Type

-

18	-	Basic Model (Model 18)

9	-	Group or Section

300	-	Drawing Serial

Type Numbers

The principal and original use of the type number was to indicate the type of aircraft of a special equipment drawing.  Some drawings employ the type numbers of 6, 7, and 8 as class numbers.

0	-	Landplane (Note:  The “O” is understood but never shown on drawings.)

1	-	Seaplane

2	-	Skiplane

3	-	Not used

4	-	Not used

5	-	Not used

6	-	Used to expand the class numbers

7	-	Used to expand the class numbers

8	-	Used to expand the class numbers

9	-	Special equipment

Class

The class numbers are used to indicate the weight classification on commercial models and functional classification of military models.

0	-	Commercial aircraft of 7200 pounds, gross weight class

1	-	Commercial aircraft of 7500 pounds, gross weight class, equipped with

approximately 300 hp engines

2	-	Commercial aircraft of 7500 pounds, gross weight class, equipped with

approximately 400 hp engines

3	-	Commercial aircraft of 7800 pounds, gross weight class

4	-	Military aircraft for foreign countries

5	-	Photographic aircraft

6	-	USAF transports

7	-	Navy transports

70	-	Navy transports

77	-	Navy transports

8	-	USAF advanced trainers

80	-	USAF advanced trainers

81	-	USAF trainer

82	-	Revised USAF transports

83	-	Navy transports

84	-	Navy advanced trainers

85	-	Revised USAF transports

9	-	Revised USAF transports

Engine Type

1	-	Wright 7 cylinder

2	-	Jacobs L-6

3	-	Wright 9 cylinder

4	-	Pratt and Whitney Wasp, Jr.

5	-	Jacobs L-5

6	-	Jacobs L-4

7	-	Continental

Group Numbers

Group numbers are allocated as follows:

0	-	Miscellaneous – three view drawings, electrical equipment, flare brackets,

lugs, rods, placards, etc

1	-	Wing Panels – Wing assembly, wing spars, wing ribs, ailerons, flap, etc

2	-	Wing Braces – Wing struts, longerons, fittings, aileron and flap brackets,

etc

3	-	Cabin Furnishings – Cabin floor and door, rug, woodwork, instrument

panel

4	-	Fuselage Structure – Fuselage spars, longerons, bulkheads, fittings

5	-	Fairing and Cowling – Nacelle, forward fuselage structures, tail structure,

etc

6	-	Tail Surfaces and Braces – Tail skins, struts, longerons, fittings

7	-	Control Installations – Flight

8	-	Alighting Gear – Wheel and brake, struts, shock absorber, etc

9	-	Power Plant – Engine, propeller, carburetor, primer, etc